| SUMI Safe Use of Mixtures Information for end-users |  |
| --- | --- |

| Title | large-scale use of construction chemicals by pouring application, indoor Risk management measures: Level I |
| --- | --- |
| Code | EFCC_SUMI_PW_L-li-13-i-I_v2 |

This document is intended to communicate the conditions of safe use for the product and should always be read in combination with the product’s Safety Data Sheet and labels.

General description of the process covered

“EFCC_SUMI_PW_L-li-13-i-I_v2” is related to the “EFCC_SWED_PW_L-li-13-i-I_v2”

The mixed material is poured onto / into the prepared substrate. A vibrator is used when underpouring large base plates. In case of small areas, rotating chains or wire loops help to make the material flow more smoothly, large-scale uses, indoors.

Operational Conditions

| Range of application / Process conditions | indoor applications |  |
| --- | --- | --- |
| Sector of use | For professional and/or industrial uses only! |  |
| Application technique | Use application tools with a long handle. |  |

Risk Management Measures (RMM)

Use gloves and safety goggles.

Training of the worker in relation to proper use and maintenance of gloves must be ensured.

For specifications of personal protection equipment and/or technical RMM see section 8 of the Safety Data Sheet.

For the disposal of product residues and waste see section 13 of the Safety Data Sheet.

Additional good practice advice

Wear working clothes covering also arms and legs to protect from splashes

Do not empty the product into drains / surface water / ground water.

Disclaimer

This document has been developed using the best knowledge currently available and the information is provided in good faith. The use of the compiled information remains with each user, in view of their respective activities (as indicated below). EFCC and Deutsche Bauchemie are under no conditions liable for any damage of any nature whatsoever resulting from the use or reliance on this paper.

In case this product contains ingredients classified as hazardous to the environment, the use has been assessed to be safe for the environment. The assessment is based on the exposure parameters that are described for the product use in the corresponding SPERCs.

Regarding end-users

Following Occupational Health and Safety (OSH) legislation, the employer of workers that use products that are assessed as safe according to the EFCC SWEDs, remains responsible for communicating any other relevant use or safety information to the employees. When developing workplace instructions for employees, the Safe Use of Mixtures Information (SUMI) should always be considered in combination with the SDS and the label of the product. The information provided in the present document should not be changed by the recipient (employer). If specific changes are needed, for example on the Operational Conditions or Risk Management Measures, the concerned recipient should contact their supplier (formulator).

Regarding formulators

This is a generic document for the purpose of communicating safe use of a product to end-users. It should not be regarded by risk assessors as an exhaustive document for evaluating safe use. Formulators are responsible to proof safe use of their individual products based on risk assessments provided by their suppliers or performed by themselves. If such risk assessment correlates completely with the EFCC SWED mentioned above, this SUMI can be used unaltered for communication to end users.

Regarding use of Pictograms  -  Conditions of use

- The SUMI Pictograms may only be used in relation to formulations for which the sector-specific “bottom-up” approach from one of the DUCC members has been followed/applied.

- Uses

  - SUMI Pictograms may be used in the SUMIs

  - SUMI Pictograms may be used in the workplace instructions that employers develop under the Occupational Safety and Health legislation

  - SUMI Pictograms may be used on company website for providing safe use information of formulations for which the sector-specific “bottom-up” approach from one of the DUCC members has been followed/applied

- The SUMI pictograms must only be used in the form supplied by DUCC and must not be used in a modified form.

- The SUMI Pictograms must not be presented as if they were the trade mark or branding of an individual company.

- The SUMI Pictograms shall not be used for any formulation product which conflicts with the “bottom-up” approach as developed by DUCC.

- The SUMI Pictograms shall not be used in any way which is misleading, deceptive or confusing, or which abuses it or may bring it into disrepute.

- The SUMI Pictograms shall not be provided in exchange of money.

- DUCC reserves the right on due notice to withdraw the right of any person or company to use the SUMI Pictograms in the event of non-compliance with these conditions.